CHAPTER 31 SERIES RESONANCE AND Q-FACTOR

Exercise 133, Page 520

1.  A coil having an inductance of 50 mH and resistance 8.0 Ω is connected in series with a 25 μF

capacitor across a 100 V a.c. supply. Determine (a) the resonant frequency of the circuit, and (b)

the current flowing at resonance.

(a) Resonant frequency,

= 142.4 Hz

(b) Current at resonance, I =  = 12.5A

2.  The current at resonance in a series R-L-C circuit is 0.12 mA. The circuit has an inductance of

0.05 H and the supply voltage is 24 mV at a frequency of 40 kHz. Determine (a) the circuit

resistance, and (b) the circuit capacitance.

(a) At resonance, resistance, R =  = 200 Ω

(b) At resonance,      i.e.

from which,                 capacitance, C =

= 316.6 pF

3.  A coil of inductance 2.0 mH and resistance 4.0  is connected in series with a 0.3 F capacitor.

The circuit is connected to a 5.0 V, variable frequency supply. Calculate (a) the frequency at

which resonance occurs, (b) the voltage across the capacitance at resonance, and (c) the voltage

across the coil at resonance.

4.  A series R-L-C circuit having an inductance of 0.40 H has an instantaneous voltage,

v = 60 sin(4000t - /6) volts and an instantaneous current, i = 2.0 sin 4000t amperes. Determine

(a) the values of the circuit resistance and capacitance, and (b) the frequency at which the circuit

will be resonant.

5.  A variable capacitor C is connected in series with a coil having inductance L. The circuit

possesses stray capacitance  which is assumed to be constant and effectively in parallel with

the variable capacitor C. When the capacitor is set to 2.0 nF the resonant frequency of the circuit

is 86.85 kHz, and when the capacitor is set to 1.0 nF the resonant frequency is 120 kHz.

Determine the values of (a) the stray circuit capacitance , and (b) the coil inductance L.

Exercise 134, Page 525

1.  A series R-L-C circuit comprises a 5 F capacitor, a 4  resistor and a variable inductance L.

The supply voltage is 100 V at a frequency of 159.1 Hz. The inductance is adjusted until the

p.d. across the 4  resistance is a maximum. Determine for this condition (a) the value of

inductance, (b) the p.d. across each component, and (c) the Q-factor of the circuit.

If the p.d. across the resistance is a maximum then the circuit is at resonance.

(a) At resonance,            i.e.

from which,  inductance, L =  = 0.20 H   or   200 mH

(b) Current at resonance,  A

P.d. across resistance,  = 100 V

P.d. across inductance,

=  = 50090 V

P.d. across capacitance,

=  = 500-90 V

(c) Q-factor =  = 50      (or use Q = )

2.  A series L-R-C circuit has a supply input of 5 volts. Given that inductance, L = 5 mH, resistance,

R = 75  and capacitance, C = 0.2 F, determine (a) the resonant frequency, (b) the value of

voltage across the capacitor at the resonant frequency, (c) the frequency at which the p.d. across

the capacitance is a maximum, and (d) the value of the maximum voltage across the capacitor.

(a) Resonant frequency,  = 5033 Hz

(b) At resonance, current, I =  = 66.667 mA

Voltage across capacitor, = 10.54 V

(or  Q =   and  = 10.54 V)

(c) The frequency at which the p.d. across the capacitance is a maximum,

f =  = 4741 Hz

(d) Maximum value of ,     = 10.85 V

3.  A capacitor having a Q-factor of 250 is connected in series with a coil which has a Q-factor of

80. Calculate the overall Q-factor of the circuit.

Overall Q-factor,  = 60.61

4.  An R-L-C series circuit has a maximum current of 2 mA flowing in it when the frequency of the

0.1 V supply is 4 kHz. The Q-factor of the circuit under these conditions is 90. Determine (a) the

voltage across the capacitor, and (b) the values of the circuit resistance, inductance and

capacitance.

5.  Calculate the inductance of a coil which must be connected in series with a 4000 pF capacitor to

give a resonant frequency of 200 kHz. If the coil has a resistance of 12 Ω, determine the circuit

Q-factor.

At resonance,            i.e.

from which,  inductance, L =

= 0.1583 mH   or  158.3 μH

Q-factor =  = 16.58

Exercise 135, Page 530

1.  A coil of resistance 10.05  and inductance 400 mH is connected in series with a 0.396 F

capacitor. Determine (a) the resonant frequency, (b) the resonant Q-factor, (c) the bandwidth and

(d) the lower and upper half-power frequencies.

2.  An R-L-C series circuit has a resonant frequency of 2 kHz and a Q-factor at resonance of 40. If

the impedance of the circuit at resonance is 30 Ω determine the values of (a) the inductance, and

(b) the capacitance. Find also (c) the bandwidth, (d) the lower and upper – 3 db frequencies, and

(e) the impedance at the – 3 dB frequencies.

(a) Q-factor =      hence,   40 =     since R = Z at resonance

from which,                inductance, L =  = 95.5 mH

(b) Q-factor =      hence,   40 =

from which,                    capacitance, C =  = 66.3 nF

(c) Bandwidth,  = 50 Hz

(d) For a Q-factor of 40 it may be assumed that the lower and upper half-power frequencies are

symmetrically displaced about the resonant frequency.

Lower – 3dB frequency = 2000 -  = 1975 Hz

Upper – 3dB frequency = 2000 +  = 2025 Hz

(e) At the half-power frequencies, I = 0.707

hence impedance, Z =

Thus, the impedance at the – 3dB frequencies, Z =  = 42.43 Ω

3.  A filter in the form of a series L-C-R circuit is designed to operate at a resonant frequency of

20 kHz and incorporates a 20 mH inductor and 30  resistance. Determine the bandwidth of the

filter.

Q-factor =  = 83.776

Since Q =   then bandwidth,  =  = 238.7 Hz

4.  A circuit consists of a coil of inductance 200 μH and resistance 8.0 Ω in series with a lossless

500 pF capacitor. Determine (a) the resonant Q-factor, and (b) the bandwidth of the circuit,

(a) Q-factor =  = 79.06

(b) Resonant frequency,  = 503.292 kHz

Since Q =   then bandwidth,  =  = 6366 Hz   or   6.366 kHz

5.  A coil of inductance 200 H and resistance 50.27  and a variable capacitor are connected in

series to a 5 mV supply of frequency 2 MHz. Determine (a) the value of capacitance to tune the

circuit to resonance, (b) the supply current at resonance, (c) the p.d. across the capacitance at

resonance, (d) the bandwidth, and (e) the half-power frequencies.

6.  A supply voltage of 3 V is applied to a series R-L-C circuit whose resistance is 12 , inductance

is 7.5 mH and capacitance is 0.5 F. Determine (a) the current flowing at resonance, (b) the

current flowing at a frequency 2.5% below the resonant frequency and (c) the impedance of the

circuit when the frequency is 1% lower than the resonant frequency.

(a) At resonance,  = 12

Current at resonance,  = 0.250 A

(b) If the frequency is 2.5% below the resonant frequency, then  = 0.025

where   Q =  = 10.206

Hence,

i.e. current at a frequency 2.5% below resonance, I =  = 0.22327.04 A

(c)    from which, impedance of the circuit when the frequency is 1% lower than

the resonant frequency, Z =

= 12(1 - j0.2041) = 12

= 12.25-11.54