CHAPTER 43 FIELD THEORY

Exercise 166, Page 699

1.  (a) Explain the meaning of the terms (i) streamline (ii) equipotential, with reference to an electric

field. (b) A field plot between two metal plates is shown below. If the relative permittivity of the

dielectric is 2.4, determine the capacitance of a 50 cm length of the system.

(a)(i)  A streamline is a line of force in an electrostatic field.

(ii) Equipotential lines are lines drawn connecting together all points within a field having equal

potentials. For more see page 643 - 646.

(b) Capacitance, C =    where m = number of parallel squares measured along each

equipotential – in above diagram, m = 11,  and n = number of series squares measured along

each streamline (or line of force) – in the diagram above, n = 5.

For a 50 cm length of the system, with  = 2.4,

capacitance, C =

= 23.4 pF

2.  A field plot for a concentric cable is shown below. The relative permittivity of the dielectric is 5.

Determine the capacitance of a 10 m length of the cable.

Capacitance, C =    where  = 5, x = 10 m, m = 15 (= number of parallel squares measured along each equipotential), and n = 4 (= number of series squares measured along each line of force).

Hence, capacitance, C =   = 1.66 nF

3.  The plates of a capacitor are 10 mm long and 6 mm wide and are separated by a dielectric 3 mm

thick and of relative permittivity 2.5. Determine the capacitance of the capacitor (a) when

neglecting any fringing at the edges, (b) by producing a field plot taking fringing into

consideration.

Total capacitance, C = farads

Exercise 167, Page 704

1.  A coaxial cable has an inner conductor of radius 0.4 mm and an outer conductor of internal

radius 4 mm. Determine the capacitance per metre length of the cable if the dielectric has a

relative permittivity of 2

Capacitance, C =  = F = 48.30 pF

2.  A concentric cable has a core diameter of 40 mm and an inner sheath diameter of 100 mm. The

relative permittivity of the dielectric is 2.5 and the core potential is 50 kV. Determine (a) the

capacitance per kilometre of the cable and (b) the dielectric stress at radii of 30 mm and 40 mm.

(b) Dielectric stress, E, at radius r,

3.  A coaxial cable has a capacitance of 100 pF per metre length. The relative permittivity of the

dielectric is 3.2 and the core diameter is 1.0 mm. Determine the required inside diameter of the

sheath.

Capacitance, C =      from which,      = 1.7794

Core radius, a = 1.0/2 = 0.5 mm     hence

i.e.

and the internal radius of the sheath, b = 0.5= 2.963 mm

from which, the inside diameter of the sheath = 2 × 2.963 = 5.93 mm

4.  A single-core concentric cable is to be manufactured for a 100 kV, 50 Hz transmission system.

The dielectric used is paper which has a maximum safe dielectric stress of 10 MV/m and a

relative permittivity of 3.2. Calculate (a) the core and inner sheath radii for the most economical

cable, (b) the capacitance per metre length and (c) the charging current per kilometre run.

(a) Core radius, a, for the most economical cable is given by:   a =

i.e.   core radius, a =  = 0.01 m   or   10 mm

Inner sheath radius, b, for the most economical cable is given by:   b = a e = 2.718 a

i.e. inner sheath radius, b = (2.718)(10) = 27.2 mm

(b) Capacitance per metre length, C, for a concentric cable is given by:

C =  = 177.9 pF

(c) Charging current =

Hence, for a 1 km run, charging current =  = 5.59 A

5.  A concentric cable has a core diameter of 30 mm and an inside sheath diameter of 75 mm. The

relative permittivity is 2.6, the loss angle is rad and the working voltage is 100 kV at

50 Hz frequency. Determine for a 1 km length of cable (a) the capacitance, (b) the charging

current, and (c) the power loss.

(a) Capacitance per metre length, C =

Hence, capacitance for a 1 km length of cable = 157.8 pF/m ×

= 157.8 nF   or   0.1578 μF

(b) Charging current, I =

= 4.957 A

(c) Power loss =

= 1239 W

6.  A concentric cable operates at 200 kV and 50 Hz. The maximum electric field strength within

the cable is not to exceed 5MV/m. Determine (a) the radius of the core and the inner radius of the

sheath for ideal operation, and (b) the stress on the dielectric at the surface of the core and at the

inner surface of the sheath.

(a) For ideal operation, the core radius, a, is given by:

Exercise 168, Page 706

1.  Two parallel wires, each of diameter 5.0 mm are uniformly spaced in air at a distance of 40 mm

between centres. Determine the capacitance of a 500 m run of the line.

Capacitance per metre length, C, of an isolated twin line is given by:

C =    since   = 1 for air

= 10.028

For a 500 m run of line, capacitance, C = 500 10.028  = 5.014 nF

2.  A single-phase circuit is comprised of two parallel conductors each of radius 5.0 mm and spaced

1.5 m apart in air. The p.d. between the conductors is 20 kV at 50 Hz, Determine (a) the

capacitance per metre length of the conductors, and (b) the charging current per kilometre run.

(a) Capacitance per metre length, C =    since for air and 1.5 m

= 1500 mm

= 4.875 pF

(b) Charging current per metre length, I =

= 30.63 × A

Hence, charging current per km length = 30.63 ××= 30.63 mA

3.  The capacitance of a 300 m length of an isolated twin line is 1522 pF. The line comprises two air

conductors which are spaced 1200 mm between centres. Determine the diameter of each

conductor.

4.  An isolated twin line is comprised of two air-insulated conductors, each of radius 8.0 mm, which

are spaced 1.60 m apart. The voltage between the lines is 7 kV at a frequency of 50 Hz.

Determine for a 1 km length (a) the line capacitance, (b) the value of the charge carried by each

wire, and (c) the charging current.

(a) Capacitance per kilometre length, C =  = 5.248 nF

(b) Charge, Q = CV =  = 36.74 μC

(c) Charging current per km length, I =

= 11.54 mA

5.  The charging current for a 1 km run of isolated twin line is not to exceed 30 mA. The p.d.

between the lines is 20 kV at 50 Hz. If the line is air insulated and the conductors are spaced

1 mm apart, determine (a) the maximum value required for the capacitance per metre length, and

(b) the maximum diameter of each conductor.

Exercise 169, Page 708

1.  Determine the energy stored in a 5000 pF capacitor when charged to 800 V and the average

power developed if this energy is dissipated in 20 s.

Energy stored in capacitor, J   or   1.6 mJ

Average power developed = = 80 W

2.  A 0.25 F capacitor is required to store 2 J of energy. Determine the p.d. to which the capacitor

must be charged.

Energy stored,

from which,   p.d., V =  = 4000 V   or   4 kV

3.  A capacitor is charged with 6 mC. If the energy stored is 1.5 J determine (a) the voltage across

the plates, and (b) the capacitance of the capacitor.

4.  After a capacitor is connected across a 250 V d.c. supply the charge is 5 μC. Determine (a) the

capacitance, and (b) the energy stored.

(a) Since Q = CV   then   capacitance, C =  = 20 nF

(b) Energy stored,  = = 0.625 mJ   or   625 μJ

5.  A capacitor consisting of two metal plates each of area 100  and spaced 0.1 mm apart in air

is connected across a 200 V supply. Determine (a) the electric flux density, (b) the potential

gradient and (c) the energy stored in the capacitor.

6.  A mica capacitor is to be constructed to have a capacitance of 0.05 F and to have a steady

working potential of 2 kV maximum. Allowing a safe value of field stress of 20 MV/m,

determine (a) the required thickness of the mica dielectric, (b) the area of plate require dif the

relative permmittivity of the mica is 5, (c) the maximum energy stored by the capacitor, and

(d) the average power developed if this energy is dissipated in 25 s.

(a) Field stress, E = , from which,

thickness of mica dielectric, d = = 0.1 mm

(b) Capacitance, C =

Hence, cross-sectional area of plate, A =  = 0.113

(c) Maximum energy stored,  = 0.1 J

(d) Average power dissipated =  = 4000 W   or   4 kW

7.  A 500 pF capacitor is charged to a p.d. of 100 V. The dielectric has a cross-sectional area of

200  and a relative permittivity of 2.4. Determine the energy stored per cubic metre in the

dielectric.

and

8.  Two parallel plates each having dimensions 30 mm by 50 mm are spaced 8 mm apart in air. If a

voltage of 40 kV is applied across the plates determine the energy stored in the electric field.

Capacitance, C =  = 1.6594 pF

Energy stored,  = = 1.328 mJ

Exercise 170, Page 712

1.  A coaxial cable has an inner core of radius 0.8 mm and an outer sheath of internal radius

4.8 mm. Determine the inductance of 25 m of the cable. Assume that the relative permeability of

the material used is 1.

Inductance of a concentric cable per metre length,

L =

Hence, for a 25 m length, inductance =  = 10.2 H

2.  A concentric cable has a core 12 mm diameter and a sheath 40 mm diameter, the sheath having

negligible thickness. Determine the inductance and the capacitance of the cable per metre

assuming nonmagnetic materials and a dielectric of relative permittivity 3.2.

Inductance of a concentric cable per metre length,

Capacitance of a concentric cable per metre length,

3.  A concentric cable has an inner sheath radius of 4.0 cm. The inductance of the cable is

0.5 μH/m. Ignoring inductance due to internal linkages, determine the radius of the core. Assume

that the relative permeability of the material is unity.

Inductance per metre length =

i.e.

from which,

i.e.                                        and     a =

and    core radius, a == 0.328 cm = 3.28 mm

4.  The inductance of a concentric cable of core radius 8 mm and inner sheath radius of 35 mm is

measured as 2.0 mH. Determine (a) the length of the cable, and (b) the capacitance of the cable.

Assume that nonmagnetic materials are used and the relative permittivity of the dielectric is 2.5.

Exercise 171, Page 714

1.  A single-phase power line comprises two conductors each with a radius of 15 mm and spaced

1.8 m apart in air. Determine the inductance per metre length, ignoring internal linkages and

assuming the relative permeability, = 1

Inductance =  = 1.915 μH/m

2.  Determine (a) the loop inductance, and (b) the capacitance of 500 m length of single-phase twin

line having conductors of diameter 8 mm and spaced 60 mm apart in air.

(a) Loop inductance, L, is given by:

(b) Capacitance, C, is given by:

3.  An isolated twin power line has conductors 7.5 mm radius. Determine the distance between

centres if the total loop inductance of 1 km of the line is 1.95 mH.

Loop inductance, L, is given by:

4.  An isolated twin line has conductors of diameter metres and spaced D millimetres apart

in air. Derive an expression for the total loop inductance L of the line per metre length.

Total loop inductance, L =      For proof of this formula, see pages 588 and 589 of

textbook

For air, = 1, distance apart of conductors = D × m, radius of each conductor =

Hence, total loop inductance, L =  =

5.  A single-phase power line comprises two conductors spaced 2 m apart in air. The loop

inductance of 2 km of the line is measured as 3.65 mH. Determine the diameter of the

conductors.

Loop inductance, L, is given by:

Exercise 172, Page 717

1.  Determine the value of the energy stored when a current of 120 mA flows in a coil of 500 mH.

What value of current is required to double the energy stored?

Energy stored in coil,  = 3.6 mJ

If the energy is doubled, then

from which, current,    or   169.7 mA

2.  A moving-coil instrument has two airgaps each 2.5 mm long and having a cross-sectional area of

8.0 . Determine the total energy stored in the magnetic field of the airgaps if the flux density

is 100 mT.

Energy stored,

3.  Determine the flux density of a uniform magnetic field if it is to have the same energy as that

established by a uniform electric field of strength 45 V/m. Assume the relative permeability of

the magnetic field and the relative permittivity of the electric field are both unity.

Energy stored in electric field,

=  =

Energy stored in magnetic field,

Hence, if                     = 8960.625

then flux density, B =  = 0.15 T

4.  A long single core concentric cable has inner and outer conductors of diameters and

respectively. The conductors each carry a current of I amperes but in opposite directions. If the

relative permeability of the material is unity and the inductance due to internal linkages is

negligible, show that the magnetic energy stored in a 4 m length of the cable is given by:

joules

Inductance of concentric cables, ignoring flux linkages,

L =  H/m

Magnetic energy stored,  =  J/m

For a 4 m length of the cable, energy stored =

=  joules

5.  1 mJ of energy is stored in a uniform magnetic field having dimensions 20 mm by 10 mm by

1.0 mm. Determine for the field (a) the flux density, and (b) the magnetic field strength.

Hence,

Hence,