CHAPTER 6 BATTERIES AND ALTERNATIVE SOURCES OF ENERGY

EXERCISE 35, Page 83

1.  Twelve cells, each with an internal resistance of 0.24  and an e.m.f. of 1.5 V are connected (a)

in series, (b) in parallel. Determine the e.m.f. and internal resistance of the batteries so formed.

(a) Total e.m.f. in series = 12  1.5 = 18 V

Total internal resistance in series = 12  0.24 = 2.88

(b) Total e.m.f. in parallel = 1.5 V

Total internal resistance in parallel =  0.24 = 0.02

2.  A cell has an internal resistance of 0.03  and an e.m.f. of 2.2 V. Calculate its terminal p.d. if it

delivers (a) 1 A,  (b) 20 A,  (c) 50 A.

(a) Terminal p.d., V = E – Ir = 2.2 – (1)(0.03) = 2.17 V

(b) Terminal p.d., V = E – Ir = 2.2 – (20)(0.03) = 2.2 – 0.6 = 1.6 V

(c) Terminal p.d., V = E – Ir = 2.2 – (50)(0.03) = 2.2 – 1.5 = 0.7 V

3.  The p.d. at the terminals of a battery is 16 V when no load is connected and 14 V when a load

taking 8 A is connected. Determine the internal resistance of the battery.

When no load is connected the e.m.f. of the battery, E, is equal to the terminal p.d., V, i.e.  E = 16 V

When current I = 8 A and terminal p.d. V = 14 V, then  V = E - Ir

i.e.                                                                                     14 = 16 - (8)r

Hence, rearranging, gives                                                 8r = 16 - 14 = 2

and the internal resistance, r =  = 0.25

4.  A battery of e.m.f. 20 V and internal resistance 0.2  supplies a load taking 10 A. Determine the

p.d. at the battery terminals and the resistance of the load.

P.d. at battery terminals, V = E – Ir = 20 – 10(0.2) = 18 V

Load resistance,  = 1.8

5.  Ten 2.2 V cells, each having an internal resistance of 0.1  are connected in series to a load of

21 . Determine (a) the current flowing in the circuit, and (b) the p.d. at the battery terminals.

E.m.f.,  E = 10  2.2 = 22V, and internal resistance, r = 10  0.1 = 1

(a) Current, I =  = 1 A

(b) P.d. at the battery terminals, V = E – Ir = 22 – (1)(1) = 21 V     (or  V = I = 21 V)

6.  For the circuits shown below the resistors represent the internal resistance of the batteries.  Find,

in each case: (i)  the total e.m.f. across PQ

(ii) the total equivalent internal resistances of the batteries.

(i)(a) Total e.m.f., E = 4 + 5 – 3 = 6 V

(b) Total e.m.f., E = 2 V

(ii)(a) Total equivalent internal resistance = 1 + 2 + 1 = 4 Ω

(b) Total equivalent internal resistance is given by:

from which,   = 0.25 Ω

7.  The voltage at the terminals of a battery is 52 V when no load is connected and 48.8 V when a

load taking 80 A is connected. Find the internal resistance of the battery. What would be the

terminal voltage when a load taking 20 A is connected?

V = E – Ir,   hence     48.8 = 52 – 80 r

from which,                80 r = 52 – 48.8 = 3.2

and internal resistance, r =  = 0.04

When I = 20 A,  terminal voltage, V = 52 – 20(0.04) = 52 – 0.8 = 51.2 V