Bird’s Electrical Circuit Theory and Technology 7th edition

ISBN 978-0-367-67222-5

www.routledge.com/cw/bird

© Taylor & Francis, 2021

All rights reserved

MULTIPLE CHOICE QUESTIONS FOR CHAPTER 16

All questions have only one correct answer

1.	The value of an alternating current at any given instant is:

[a]	a maximum value

[b]	a peak value

[c]	an instantaneous value

[d]	an r.m.s. value

2.	An alternating current completes 100 cycles in 0.1 s. Its frequency is:

[a]	20 Hz

[b]	100 Hz

[c]	0.002 Hz

[d]	1 kHz

3.	In Figure 1, at the instant shown, the generated e.m.f. will be:

[a]	zero

[b]	an r.m.s. value

[c]	an average value

[d]	a maximum value

Figure 1

4.	The supply of electrical energy for a consumer is usually by a.c. because:

[a]	transmission and distribution are more easily effected

[b]	it is most suitable for variable speed motors

[c]	the volt drop in cables is minimal

[d]	cable power losses are negligible

5.	Which of the following statements is false?

[a]	It is cheaper to use a.c. than d.c.

[b]	Distribution of a.c. is more convenient than with d.c. since voltages may be readily altered

using transformers

[c]	An alternator is an a.c. generator

[d]	A rectifier changes d.c. to a.c.

6.	An alternating voltage of maximum value 100 V is applied to a lamp. Which of the following

direct voltages, if applied to the lamp, would cause the lamp to light with the same

brilliance?

[a]	100 V

[b]	63.7 V

[c]	70.7 V

[d]	141.4 V

7.	The value normally stated when referring to alternating currents and voltages is the:

[a]	instantaneous value

[b]	r.m.s. value

[c]	average value

[d]	peak value

8.	State which of the following is false. For a sine wave:

[a]	the peak factor is 1.414

[b]	the r.m.s. value is 0.707 x peak value

[c]	the average value is 0.637 x r.m.s. value

[d]	the form factor is 1.11

9.	An a.c. supply is 70.7 V, 50 Hz. Which of the following statements is false?

[a]	The periodic time is 20 ms

[b]	The peak value of the voltage is 70.7 V

[c]	The r.m.s. value of the voltage is 70.7 V

[d]	The peak value of the voltage is 100 V

10.	An alternating voltage is given by V=100 sin (50πt - 0.30)V. Which of the following

statements is true?

[a]	The r.m.s. voltage is 100 V

[b]	The periodic time is 20 ms

[c]	The frequency 25 Hz

[d]	The voltage is leading v = 100 sin 50π by 0.30 radians

11.	The number of complete cycles of an alternating current occurring in one second is known

as:

[a]	the maximum value of the alternating current

[b]	the frequency of the alternating current

[c]	the peak value of the alternating current

[d]	the r.m.s. or effective value

12.	A rectifier conducts:

[a]	direct currents in one direction

[b]	alternating currents in one direction

[c]	direct currents in both directions

[d]	alternating currents in both directions