Chapter 1

Principles of 2D and 3D ultrasound

Q1: What is the frequency of ultrasound (US) waves?

A)	≥10,000 Hz

B)	≥20,000 Hz

C)	≥30,000 Hz

D)	≥40,000 Hz

Answer: B

Sound waves with frequencies above the audible range of ≥ 20,000 Hertz (Hz) comprise US. Sound is a mechanical wave that propagates in a medium through molecular interaction by successive molecular compressions and rarefactions.

Q2: What type of wave does ultrasound experience in solids?

A)	Transverse waves

B)	Longitudinal waves

C)	Both transverse and longitudinal waves

D)	Neither transverse nor longitudinal waves

Answer: C

Solids, such as biological tissues, can experience both transverse and longitudinal waves.

Q3: What is sonoelasticity imaging used for?

A)	To measure the speed of ultrasound in a medium

B)	To measure the speed of light in a medium

C)	To measure the viscoelastic properties of biological tissues

D)	To measure the acoustic properties of a medium

Answer: C

Research suggests that shear waves may become clinically useful in characterizing the viscoelastic properties of biological tissues, and in sonoelasticity imaging and dynamic elastography.

Q4: What is the typical frequency of ultrasound usually used in medical imaging?

A)	2-5 MHz

B)	2-10 MHz

C)	2-15 MHz

D)	2-20 MHz

Answer: C

The frequency of US used for medical imaging ranges from 2 to 15 MHz, although it can be higher for some special imaging techniques.

Q5: What is the time interval of a cycle inversely related to?

A)	Duration

B)	Frequency

C)	Source

D)	Energy

Answer: B

The time interval, or the duration of a cycle, is the period (τ), which inversely relates to the frequency (f=1/τ), thus a higher frequency implies a shorter period at each propagation speed.

Q6: How does ultrasound interact with a medium?

A)	It is reflected or scattered

B)	It is absorbed and then reemitted

C)	It is absorbed and then reflected

D)	It is scattered and then reemitted

Answer: A

As US interacts with a medium, there is an absorption of most of the transmitted energy while the rest reflects or scatters. When US interacts with structures smaller than the wavelength, non-specular reflection or scattering.

Q7: What is typical of parenchymal tissue smaller than the wavelength when it interacts with ultrasound?

A)	Absorption of the ultrasound

B)	Reflection of the ultrasound

C)	Non-specular reflection or scattering

D)	Reemission of the ultrasound

Answer: C

As US interacts with a medium, there is an absorption of most of the transmitted energy while the rest reflects or scatters. When US interacts with structures smaller than the wavelength, non-specular reflection or scattering.

Q8: What is the main effect of ultrasound interacting with a medium?

A)	Reflection

B)	Scattering

C)	Absorption

D)	Reemission

Answer: C

As US interacts with a medium, there is an absorption of most of the transmitted energy while the rest reflects or scatters. When US interacts with structures smaller than the wavelength, non-specular reflection or scattering.

Q9: What is the range of pixel values for a grayscale ultrasound image?

A)	0 to 255

B)	0 to16

C)	0 to 24

D)	0 to32

Answer: A

Simple US images appear in grayscale, with a pixel value ranging from 0 to 255 (8 bits).

Q10: Which modality has the highest temporal resolution?

A)	2D imaging

B)	3D imaging

C)	Color Doppler imaging

D)	Mmode

Answer: D

Refreshing of B-mode images with a frame rate of 30–90 images/s limits visualizing rapidly moving structures, such as heart valves. Thus, some prefer M-mode because of the higher temporal resolution, which can be below 1 millisecond.